| Test Parameters | Specifications | Reference | Test Results |
| --- | --- | --- | --- |
| Description | Clear, colourless, syrupy viscous liquid; odourless; sweet taste | BP-2024 | Clear, colourless, syrupy viscous liquid; odourless; sweet taste |
| Identification (IR) | IR spectrum matches reference standard or Chemical Reaction Test. | BP-2024 | Complies |
| Solubility | Miscible with water and with alcohol | BP-2024 | Miscible with water and with alcohol |
| Acidity or Alkalinity | Passes test (0.1N NaOH or HCl with phenolphthalein) | BP-2024 | 0.10 ml of 0.1N sodium hydroxide consumed. |
| Refractive Index (20°C) | 1.470 – 1.477 | BP-2024 | 1.470 |
| Specific Gravity (20°C) | 1.249 – 1.258 | BP-2024 | 1.261 |
| Chloride (Cl⁻) | ≤ 10 ppm | BP-2024 | Complies |
| Aldehydes | The absorbance of sample is not more than the standard solution | BP-2024 | Complies |
| Esters | No turbidity on ethanol test | BP-2024 | Complies |
| Sugars / Reducing Substances | No red colour with Fehling’s solution (passes as no reducing sugars) | BP-2024 | Complies |
| Water Content | ≤ 0.5 % (w/w) | BP-2024 | 0.091 |
| Halogenated Compounds | Any opalescence in the solution is not more intense than the standard solution. | BP-2024 | Complies |
| Assay (C₃H₈O₃ Content) | 98.0% to 101.0% (w/w) on an anhydrous basis | BP-2024 | 99.5 |
| Sulphated Ash (%) | ≤ 0.01 on 5.0 g | BP-2024 | 0.004 |

Storage Conditions:

- Recommended temperature for storage: Below 40°C (preferably between 15°C and 30°C). Avoid extreme temperature fluctuations.

- Protection from Light & Contaminants: Store in a well-ventilated, dry place, away from direct sunlight. Prevent contamination with water, dust and/or other chemicals.

- Glycerine IP has a shelf life of over 24 months, but it may be re-tested and extended based on quality review and stability data. It should be stored in a tightly closed, light-resistant container, under dry conditions, following GMP guidelines.